IDENTIFICATION

Product Code:	Maindec 08-D1GB-D

Product Name:	PDP-8, 8I, 8S Extended Memory Control

Date Created:	May 5, 1968

Maintainer:	Diagnostics Group

I.	Abstract

This program tests the Extended Memory Control logic for proper operation. It may be used with a PDP-8, 8I, or 8S equipped with a minimum of 4K of extended memory. The program exercises and tests the control IOT's; the ability to reference all fields from field 0; program interrupt and interrupt inhibit; auto-indexing in each field, and a special test for the PDP-8I which tests the presence of a false memory pulse when a non-existent memory field is referenced.

Errors encountered during running will result in a program halt. The halt locations are labeled, and the error may be identified by referencing the program listing or table of error halts.

- Requirements

2.1	Equipment

A standard PDP-8, 8I or 8S equipped with an Extended Memory Control, and at least 4K of extended memory.

2.2	Storage

The program requires 1726(8) locations of code memory. The program must reside in memory field 0 only.

2.3	Preliminary Programs

All programs for a basic PDP-8, 81 or 8S must have been previously run successfully.

- Loading Procedure

3.1	Method

The program must be loaded with the Binary loader.

- Turn off the Teletype reader.

- Set the SWITCH REGISTER to 7777.

- Press LOAD ADDRESS, and then START.

- Place the program tape in the reader and turn on the reader.

- When the program has been loaded, stop the computer, turn off the reader, and remove the tape.

Page 2

- Starting Procedure

4.1	Starting Addresses

The starting address is 0200(8).

4.2	Control Switch Settings

SR 8 must be on a 1 if a PDP-8I is being used. Otherwise, on a 0 for a PDP-8 or 8S.

SR 9, 10 and 11 must contain an octal value equal to the number of EXTENDED memory fields available (1 to 7 octal). Note that field 0 is not to be included in this value.

4.3	Operator Action

With the program in memory, set the SWITCH REGISTER to 0200 octal.

Press LOAD ADDRESS.

Set SR 8 to a 1 if a PDP-8I is being used. Otherwise, set SR 8 to a 0.

Place the octal number of EXTENDED memory fields available in SR 9, 10 and 11. This value may vary from 1 to 7 only.

Press START.

The program will run until an error is detected, or stopped by the operator.

The TTY bell is rung once after one complete pass of the program.

- Operating Procedure

See section 4.2

5.2	Subroutine Abstracts

Refer to the program listing for descriptions of each test, and the method of testing.

5.3	Operator Action

See section 4.3

Page 3

6.	Errors

6.1	Error Halts and Description

Table of Error Halts

| C (MA) |  | Tag Description |  |  |
| --- | --- | --- | --- | --- |

CDF and RDF Tests

206	E 1	CDF 0 or RDF failed.

217	E 2	CDF 7 or RDF failed.

234	E 3	CDF 1 or RDF failed.

245	E 4	CDF 2 or RDF failed.

262	E 5	CDF 3 or RDF failed.

273	E 6	CDF 4 or RDF failed.

310	E 7	CDF 5 or RDF failed.

321	E 8	CDF 6 or RDF failed.

DF, IB and SF Tests

341	E 9	RIB or ION failed.

351	E 10	DF not cleared, or no interrupt.

360	E II	RIB or SF failed. (DF 1)

410	E 12	DF not cleared, or no interrupt.

417	E 13	RIB or SF failed. (DF 2)

427	E 14	DF not cleared, or no interrupt.

436	E 15	RIB or SF failed. (DF 3)

452	E 16	DF not cleared, or no interrupt.

462 461	E 17	RIB or SF failed. (DF 4)

Page 4

471	E 18	DF not cleared or no interrupt.

500	E 19	RIB or SF failed. (DF 5)

514	E 20	DF not cleared, or no interrupt.

523	E 21	RIB or SF failed. (DF 6)

533	E 22	DF not cleared, or no interrupt.

542	E 23	RIB or SF failed. (DF 7)

DCA I and TAD I Tests

653	E 24	DCA I or TAD I to an extended field failed.

The DF indicators equal the current field under test. The AC contains the data as

read from location 7000 of the extended field. The halt occurred due to the data read and the current data field not being equal. Each extended field should contain its field number in location 7000.

CIF, IB and SF Tests

Program interrupt is enabled for these tests. A CIF is issued, followed by an

ION and a JMP. The IF should always equal 0, because of the interrupt occurring after the JMP instruction. A HLT is in location 1 of each intended field in case the IF does get set. The TTY flag is used for interrupts.

| 710 | E 25 | No interrupt, or inhibit interrupt failed. |
| --- | --- | --- |
| 717 | E 26 | CIF 1. The IB or SF failed. The AC = C(IB). |
| 733 | E 27 | No interrupt or inhibit interrupt failed. |
| 742 | E 28 | CIF 2. The IB or SF failed. The AC = C(IB). |
| 756 | E 29 | No interrupt or inhibit interrupt failed. |
| 765 | E 30 | CIF 3. The IB or SF failed. The AC = C(IB). |
| 1004 | E 34 31 | No interrupt, or inhibit interrupt failed. |
| 1013 | E 32 | CIF 4. The IB or SF failed. The AC = C(IB). |

|  |  | Page 5 |  |  |
| --- | --- | --- | --- | --- |
| 1027 | E 33 |  | No interrupt, or inhibit interrupt failed. |  |
| 1036 | E 34 |  | CIF 5. The IB or SF failed. The AC = C(IB). |  |
| 1052 | E CIF IOT |  | No interrupt, or inhibit interrupt failed. |  |
| 1061 | E 36 |  | CIF 6. The IB or SF failed. The AC = C(IB). |  |
| 1075 | E 37 |  | No interrupt, or inhibit interrupt failed. |  |
| 1104 | E 38 |  | CIF 7. The IB or SF failed. The AC = Cended |  |

Interrupt Inhibit Test

A subroutine is placed in each extended field to ensure that program interrupt

is inhibited after a CIF IOT, and is enable after a JMP instruction. The routine is in one field at a time; the contents all other extended fields will equal 0000. The routine is described on the program listing as the "Extended Field Test Routine," and is tagged EXFLD.

The test routine is entered at location1174 in the extended field. This location contains a CIF XX I0T, where XX equals the extended field number. Location 1175 contains an ION 10T. Locations 1176 to 7776 contain all 0's. Location 7777 contains a JMP I 12. The routine, therefore, issues a CIF, ION, and JMP I 12 sequence. Program interrupt should be inhibited until after the JMP I 12 at location 7777. An error halt occurs in field 0 if an interrupt occurs between locations. 1176 and 7777. Location 12 contains 1175 (E 40 in field 0), and will auto-index to 1176.

| 1175 | E 40 | CIF or interrupt failed. The DF and IF should equal an extended field. |
| --- | --- | --- |
| 1203 | E 41 | The DF was not cleared after the interrupt. All other functions worked properly. |
| 1221 | E 42 | RMF or SF failed. The SF register should have saved the extended field number after interrupt. The AC = C(I.B.) after an RMF. |
| 1235 | E 43 | All functions worked, but the PC did not equal location E 40 + 1 after the interrupt in the extended field failed. The AC = contents of location 0, field 0. |
| 4 | E 44 | Location 4 in the extended field. The interrupt went to this field instead of field 0, or the JMP I 12 at location 7777 was not executed. Also, make sure interrupt was enabled in location 1175 in the extended field. |

Page 6

10	E 45	Location 10 in the extended field. The

JMP 1 12 at location 7777 was not executed, or interrupt failed.

| 422 1420 E 45A 7000 | No program interrupt occurred. Press CONTINUE tey try again. Memory field 1 halt. An interrupt in field 0 was followed by a CIF 10 I0T, and an RMF. The RMF should have restored the IB to field 0. The SF and IB were OR'd together, resulting in the IF being set to field 1, after the JMP instruction at location 1430. Restart from 1400 to repeat the test. |
| --- | --- |

|  | Auto-Index Test |  |  |
| --- | --- | --- | --- |
| The subroutine labeled "Auto-Index Test" on the listing is placed in each extended field. Auto-index registers 10 through 17 in each field are tested. All of memory not occupied by the subroutine in set to 0. The error halts tagged E 46 through E 53 will occur in the extended field if an auto-index register fails. The DF and IF indicators will display the current field being tested. |  |  |  |

1524 1522	E46	Index register 10 failed.

1527 1525	E 47	Index register 11 failed.

1532 1530	E 48	Index register 12 failed.

1535 1533 	E 49	Index register 13 failed.

1540 1536 	E 50	Index register 14 failed.

1453 1541	E 51	Index register 15 failed.

1546 1544 	E 52	Index register 16 failed.

1551 1547 	E 53	Index register 17 failed.

Non-Existent Memory Test

This is the last test performed, and is included for PDP-8I's only. The test makes sure that a false memory done pulse in generated when the DF is set to a non-existent memory field. If the PDP-8I being used is equipped with the maximum of 32 K of

Page 7

code memory, the program automatically skips this test and restarts at location 200. SR 8 on a 0 will cause the program to always skip this test.

The test also makes certain that the correct data is deposited in the AC when a non-existent field is referenced. This data must always equal 0000 or 7777 octal, depending on the number of extended fields existing. For example, if the PDP-8I is equipped with fields 0, 1, 2 and 3, any reference with a TAD I to fields 4 through 7 should result with 7777 octal in the AC. If equipped with fields 0, 1 and 2, a TAD I to field 3 should result with 0000 octal in the AC, and referencing 4 through 7 will result with 7777 octal in the AC. In other words, referencing the lowest order non-existent field, when the total number available is add, will result with 0000 in the AC. Referencing all other non-existent fields will result with 7777 in

the AC. When the total number available is even, referencing any non-existent field will result with 7777 in the AC.

The only legal halts in this test, are at locations 1700 and 1725. If the computer halts at any other location, the false memory done pulse probably was not generated.

The false memory done pulse is not generated when a CIF to a non-existent field is attempted.

1700	E 54	All 0's should have been deposited

in the AC, or an existing field was referenced. Make sure the proper value is in SR 9-11. The

number of extended  fields available must be in SR 9-11.

1725	E 57	All l's should have been deposited

in the AC, or an existing field was referenced. Make sure the proper value is in SR 9-11.

6.2	Error Recovery

Press CONTINUE to repeat the failing test. Place a NOP in the error halt location to loop on a failing test. Restart from 1400 after a halt at 7000 in field 1.

7.	Restrictions

7.1	Starting Restrictions

None

Page 8

7.2	Operating Restrictions

The number of extended memory fields available must be in SR 9-11 before starting from location 200.

8.	Miscellaneous

8.1	Execution Time

Running time is dependent on the amount of extended memory for testing, and on whether the processor being used is a PDP-8, 8I or 8S. The TTY bell will ring once for each pass of the program.

- Program Description

The program exercises all I0T's associated with the Extended Memory Control logic, plus the ability to reference extended fields from field 0, and vice-versa. Each test is looped 4096 times before initiating the next test. A switch option is provided to skip or execute a non-existent memory test for the PDP-8I.

The individual test routines and error halts are commented on the program listing as an aid to trouble-shooting. Section 6 contains a Table of Error Halts which also may be referenced.

- Listing