IDENTIFICATION

Product Code:	MAINDEC-8/I-DO1C-D

Product Name:	Instruction Test 1

Date Created:	December 10, 1969

Maintainer:	Diagnostic Group

MAINDEC-8/I-DO1C-D

- ABSTRACT

This is a diagnostic program for testing the AND, TAD, and OPERATE instructions of the PDP-8I ONLY.

- REQUIREMENTS

2.1	Equipment

A PDP-8I equipped with Teletype.

2.2	Storage

This program uses locations 0000-4421. The Binary Loader must be stored in the last memory page.

2.3	Preliminary Programs

- LOADING PROCEDURE

3.1	Method

The standard Binary Loader is used.

- STARTING PROCEDURE

4.1	Control Switch Settings

SR = 7777

4.2	Starting Address

0144

4.3	Operator Action

- Set SR to 144 and press LOAD ADDRESS.

- Set SR to 7777 and push START.

- Program will halt. MA=146

- If AC = 0000 an error has occurred.

- Push CONTINUE. Program will run until stopped or until an error is encountered.

MAINDEC-8/I-D01C-D

- OPERATING  PROCEDURE

Same as section 4.

- ERRORS

6.1	 Error Halts and Description

See listing.

6.2	Error Recovery

If it is desired to loop on the test causing the error, replace the operate instruction preceding the HLT with a JMP to the beginning of the test.

- RESTRICTIONS

7.1	Starting Restrictions: None

7.2	Operating Restrictions: None

- MISCELLANEOUS

8.1	Execution Time

256 program loops per second.

- PROGRAM DESCRIPTION

The program consists of many individual tests. In most cases each test checks only one function and uses previously tested functions as a basis for the test. The program counter action is strictly sequential except for SKIPS. The only JMP instructions used are at the end of the program.

The sequence of testing is as follows:

- Test all operate functions individually except for rotates.

- Test TAD instruction.

- Test ROTATE functions.

- Group 1 operate instruction test. All possible combinations.

2

MAINDEC-8/I-DO1C-D

- Group 2 operate instruction test. All possible combinations.

- Test AND instruction.

The Teletype bell rings after each 128 program loops.

10.	LISTING

MAINDEC EVALUATION REQUEST

After sufficient familiarization with the operation and documentation of this MAIN DEC, please indicate your assessment of the following areas and return this form to Digital Equipment Corporation.

IDENTIFICATION: MAINDEC NO 	Program Title

USAGE: Used by: Field Service	Production	Other	[]

Frequency of Usage: Daily []	Weekly []	Monthly []

SUGGESTIONS FOR IMPROVEMENT

- Are the program loading and operating instructions: clear?[], incomplete?[], difficult to fol low ?[]

- Do the error reports and program documentation provide sufficient diagnostic information: in all cases ?[], in most cases ? [], in very few cases []. Suggestions for improvement:

- Is the program effective in isolating malfunctions: in all cases ?[], in most cases?[], in very few cases? []. Would additional Scope loops or Switch Register control be helpful ? Suggestions for improvement:

- Does the program ever fail to detect malfunctions exposed by other software?

Were Margins used?	Please describe malfunction in detail:

- Does the program ever report non-existant malfunctions?	
Please indicate erroneous report and any pertinent operating conditions:

- Does this MAINDEC ever expose malfunctions in the Central Processor or other peripheral units not detected by the appropriate MAINDEC?	
Please describe malfunction and MAINDEC(S) used:

- Does the document provide a general understanding of the functional programming requirements of the system? Good[], Fair[],	None []	Would a general description of programming re-

quirements increase t e effectiveness of this AINDEC?

Remarks:

Fold Here

Do Not Tear Fold Here and Staple

|  | FIRST CLASS PERMIT NO. 33 MAYNARD, MASS. |
| --- | --- |
| BUSINESS REPLY MAIL NO POSTAGE STAMP NECESSARY IF MAILED IN THE UNITED STATES | IIIIM111111111111111=111111=1 |

| Postage will be paid by: | Digital Digital Equipment Corporation Diagnostic Programming Group 146 Main Street, Building 12 Maynard, Massachusetts 01754 |  |  |
| --- | --- | --- | --- |
|  |  | 111111•111•11•111111111111n1 1111111111111111111n1111 111111111111111•11111111111111111111Eli |  |