IDENTIFICATION

I PDP-8

LIBRARY

Product Code:	MAINDEC-8I-D02B-D

Product Name:	Instruction Test 2

Date Created:	March 21, 1968

Maintainer:	Diagnostic Group

Author:	H. Horowitz

- ABSTRACT

This program is an extensive test of autoindexing, indirect addressing, and the DCA instruction for the PDP-8I It also offers minimal testing for interrupt and the AND, TAD, ISZ, JMP and JMS  instructions.

- REQUIREMENTS

2.1	Equipment

PDP-8I, 8S equipped with Teletype

2.2	Storage

The program occupies memory locations 0000-4021. The Binary Loader must be stored in the last memory page.

2.3	Preliminary Programs

It is assumed that Maindec 8I-D01(n) has been run successfully.

- LOADING PROCEDURE

3.1	Method

The standard Binary Loader is used.

- STARTING PROCEDURE

4.1	Control Switch Settings (None)

4.2	Starting Address

201

4.3	Operator Action

- Set the SWITCH REGISTER to 201. Press LOAD ADDRESS.

PUSH START. A programmed halt occurs when an error is encountered.

1

5.	OPERATING PROCEDURE

Same as Section 4.

- ERRORS

6.1	Error Halts and Description

See listing.

6.2	Error Recovery

If it is desired to loop on the test causing the error, replace the operate instruction preceding the HLT with a JMP to the beginning of the test.

- RESTRICTIONS

All peripheral devices must be functioning to a degree that the processor START button will clear their flags and no FLAGS will turn on unless authorized,

- MISCELLANEOUS

8.1	Execution Time

Five program loops per second.

- PROGRAM DESCRIPTION

The program is a series of many small individual tests. Each test is performed using previously checked functions to validate the test. When an error is encountered, the operator should insert a jump instruction into memory to cause the program to loop on the failing test. The problem can then be found by using the oscilloscope.

The individual program tests are grouped into the following classifications and are performed in the order shown:

- DCA Test

- Indirect Addressing Test

using TAD

2

- using AND

| 3. Autoindex Test 3 - using AND - using DCA - using TAD 4. Interrupt Test - following NOP - following TAD - following AND - following DCA - following ISZ - following JMP - following JMS - following CLA - following TSF 5. Check for unauthorized Autoindexing At the completion of 40 program loops the bell rings to assure the operator that the Program is functioning properly. |  |
| --- | --- |

10.	LISTING

MAINDEC EVALUATION REQUEST

After sufficient familiarization with the operation and documentation of this MAIN DEC, please indicate your assessment of the following areas and return this form to Digital Equipment Corporation.

IDENTIFICATION: MAINDEC NO 	Program Title

USAGE: Used by: Field Service 0 Production 0 Other

FreqDency of Usage: Daily	Weekly 0 Monthly 0

SUGGESTIONS FOR IMPROVEMENT

- Are the program loading and operating instructions: clear?0, incomplete? 0, difficult to follow?D

- Do the error reports and program documentation provide sufficient diagnostic information. in all cases ?0, in most cases? 0, in very few cases ID . Suggestions for improvement:

- Is the program effective in isolating malfunctions: in all cases?0, in most cases?[1, in very few cases? n. Would additional Scope loops or Switch Register control be helpful ?	
Suggestions for improvement:

- Does the program ever fail to detect malfunctions exposed by other software?

Were Margins used?	Please describe malfunction in detail:

- Does the program ever report non-existant malfunctions?	
Please indicate erroneous report and any pertinent operating conditions:

- Does this MAINDEC ever expose malfunctions in the Central Processor or other peripheral units not detected by the appropriate MAINDEC 	
Please describe malfunction and MAINDEC(S) used:

- Does the document rovide a general understanding of the functional programming requirements of the system? Good	, Fair,	None n. Would a general description of programming re-

quirements increase t e effectiveness of this AINDEC 7

Remarks:

Fold Here

Do Not Tear - Fold Here and Staple

BUSINESS REPLY MAIL

NO POSTAGE STAMP NECESSARY IF MAILED IN THE UNITED STATES

| Postage will be paid by: | Digital |  |
| --- | --- | --- |

Digital Equipment Corporation Diagnostic Programming Group 14A Main ctrPPt, Building 12 Maynard, Massachusetts 01754